DEPARTMENT OF ELECTRICAL AND COMPUTER ENGINEERING

EECE 340 -  Signals and Systems

Signal Analysis, Sampling, Reconstruction, Filtering 
and Application to Real-Word Signals using Fourier Techniques
 			    and Signal Processing Theory

Application to ECG signals

Course Instructor: Dr. Hadi Sarieddeen

By: Dina Abi Akar

ID: 202400103

Email: dza03@mail.aub.edu

Overview of the Project:

In this project, I aim to demonstrate a comprehensive understanding of the digital signal processing fundamentals, including a theoretical foundation, practical implementation using several methods, real-world application, system design insights, and an additional part showing advanced analysis and how we can optimize parameters based on data variability.

I chose an ECG signal to apply the functions I created in sections 1 and 2 to. This report is sectioned into parts in accordance with the project requirements to clearly explain the implementation.

1.Fourier-Based Signal Representation:

1.1 Finite Fourier Series Approximation:

This signal analysis tool is usually used to approximate functions in a finite range by using the first     few modes according to the equation:                                                        This equation is implemented in MATLAB with the function signature function [xhat,ck]=ffs(xt,t,n,T). Where the inputs are:

- xt: The original, time-limited signal.

- t: The time vector.

- N: The number of harmonics.

- T: The period to assume.

We define our interval dt as t(2)-t(1), a parameter L that corresponds to the length of out time vector, a variable ck that creates a matrix (MATLAB property ) to store our Fourier coefficients in, and a variable k that goes from -n to n.

We then, first loop through the values of k and calculate our coefficients, and second loop through the values of k to calculate our FFS and finally output the real part of it.

Test on Gaussian Pulse:

We test our signal on a Gaussian pulse by setting the number of harmonics to a value like 1000, and a time vector in steps of 0.001 for smooth plotting. We then create our sine and gaussian signals and apply our FFS to both of them. I used GitHub resources to know how to plot graphs in this and the rest of the code for better visualization of what is happening. We also measure the error according to the given function in the project requirements PDF.

1.2 Fourier Transform and Inverse:

Another continuous signal analysis tool is the Fourier Transform. We create the signature function [f, xf, W] = ftr(t, xt, T) and set mostly the same parameters we set for our Fourier series implementation in addition to a frequency interval df=1/T. This is because of the principle that a signal cannot be localized in both time and frequency, which is why the signal needs to behave inversely to the period in the frequency domain. We also define a range W=1/dt, and a frequency vector that spans over a period of the time vector. The CTFT is then computed by looping over the frequency vector and using the CTFT formula                                   .

The inverse is calculated similarly according to the formula:                                        .

Test on Gaussian Signal, Sinc and Rect FT pair:

I tested my signals and created plots for both the original and reconstructed signals through CTFT as shown in the zip file, video and PowerPoint.

2.Sampling, Reconstruction, and Aliasing:

2.1 Sampling:

This part of the project now shifts the focus to a discrete-time context. Sampling is an extremely important tool since technological tools cannot process a continuous waveform and have to compensate by taking samples at appropriate rates that would allow them to reconstruct or simulate the actual signal.

We implement a sampling function with the signature function [t_sample, x_sample] = sample(t, xt, fs) and apply it to x(t) = cos(2πf1t) + 0.5 cos(2πf2t) with f1 = 3Hz and  f2 = 7Hz. We then try different sampling rates:

- Undersampling: fs = 0.5fmax and fs=fmax

- Nyquist rate: fs = fmax

Plots are also produced for visualization.

2.2 Reconstruction:

After sampling, we attempt to reconstruct the signals using sinc interpolation. We create a function with the signature function xrcon = reconstruct(t, x_sample, fs). After producing the plots we notice how the sampling rates majorly affect the accuracy of reconstruction of the signal, with the Nyquist rate of twice the maximum spread producing the best results.

2.3 Aliasing and Ambiguity:

We first set up our original signal x_orig = cos(2*pi*f_orig*t_demo) and pick an original frequency of 5Hz.  We then use our sampling function with fs=8 Hz. The key idea in aliasing is that multiple cosine frequencies produce the same samples if falias=∣forig±mfs​∣  for any integer m. We generate 3 aliases:

- falias= |5-8|=3 Hz.

- falias = 5+8= 13Hz. (Higher harmonic; oversampling)

- falias= 2*8-5= 11 Hz (Nyquist zone)

After plotting we realize:

- Alias 1 overlays the same sampled points because sampling causes these two signals to be indistinguishable at 8 Hz.

- Alias 2 again produces the same samples.

- Alias 3 produces the same samples as well.

The problem is that these samples alone cannot tell us what the original frequency was. This causes aliasing ambiguity which means that many different signals look the same after sampling. The solution to this is bandlimiting. If we know the original signal is bandlimited to less than the Nyquist rate, we know that the only valid frequency is below 8/2=4 giving us the 3Hz as the only acceptable answer. However, the chosen signal of 5Hz is not bandlimited and will hence produce aliasing. This can be resolved by applying a filter to ensure no frequencies above 4Hz are processed.

2.4 Noise Robustness:

In this section, we examine the effect of white Gaussian noise on the accuracy of signal reconstruction from sampled data. The test signal is a sum of two cosines: x(t)=cos(2π⋅3t)+0.5cos(2π⋅7t), which contains two frequency components at 3 Hz and 7 Hz.

The code tests different sampling rates relative to the highest frequency to assess the effects it has on robustness. For each sampling rate, noise is introduced at four different noise levels. This process involves sampling, adding noise, then reconstructing via sinc interpolation. We then asses the mean squared error between the original and reconstructed signal to study the effects of the sampling rates on accuracy. The plots confirm that increasing the sampling rate improves noise robustness, because it averages out noise. However, at high sampling rates, increasing noise levels can also degrade reconstruction quality. This reinforces the importance of noise control in systems.

2.5 Connecting Fourier Series and Transforms:

In this part, we look at how the Fourier Series and the Fourier Transform are connected. We use a Gaussian pulse as our test signal  and calculate the Fourier Series coefficients using 50 harmonics over a period of 2 seconds. Then, we compute the Fourier Transform of the same signal to see its full frequency content.

To show the connection between these two methods, we pick out the Fourier Transform values at the harmonic frequencies (at k/T) and compare them with the Fourier Series coefficients. We generate two plots: one showing the size of the Fourier Series coefficients, and another showing the sampled points from the Fourier Transform at those same frequencies.

The theory says that the two are related by:

X(k/T)=T⋅ck

We check this by taking the ratio of the two and printing the average, which should be about 2 (since T=2). The results confirm the theory: the Fourier Series give us samples of the Fourier Transform at regular frequency steps. This shows how Fourier Series and Fourier Transforms are part of the same bigger picture, where one is for periodic signals, and the other for non-periodic ones.

3.Application and Analysis:

This part of the project gives us a tangible grasp on what the preceding sections have done. I chose to work on an ECG signal in this project. The original plan was to use open-source ECG data from renowned medical databases. However, I faced the challenge of uploading .dat files to MATLAB and resorted to using a code for synthesizing ECGs, the developer of which I have referenced below. As an addition to my project, I altered this code to produce tachycardic rhythms. Tachycardia is an abnormally rapid heart rate, and in terms of our course is a signal with an abnormally high frequency. After altering the code to produce an abnormal rhythm, I implemented an optimal sampling rate function that analyzes the data and makes the right sampling decisions.

3.1 Filtering and System Design:

Filters are extremely important in signals and systems. If we take ECGs for example, the raw data contains noise from blood flow, muscle movement, electric signals in the machine, etc. We filter the data to study our signals of interest without having to worry about unrelated errors. The filter function creates a finite impulse response (FIR) filter using the windowed sinc function since its Fourier transform is an ideal rectangular filter. A Hamming filter is applied to smooth out the sharp edges of the sinc function (Also looked up from internet resources referenced below). The choice of 40Hz is based specifically on ECG signals, since it retains essential cardiac information while filtering out other noise. The Z-transform analysis reveals that the FIR filter is inherently stable (all poles at the origin) but non-causal since it requires a delay for real-time implementation.

3.2 Sampling and Reconstruction with Filtered Signal:

We sample both original and filtered signals at various rates and reconstruct them to demonstrate the filtering, sampling rate, and reconstruction quality.

ECG signals (both original and filtered) are sampled at multiple rates:

  - Below Nyquist rate (under sampling)

  - At Nyquist rate (critical sampling)

  - Above Nyquist rate (oversampling)

  - At an "optimal" rate determined by spectral analysis

The sample.m function converts continuous signals to discrete samples. The reconstruct.m function employs sinc interpolation to recover the continuous signal. Reconstruction error is then calculated and visualized for each sampling rate, and frequency-domain analysis shows how filtering before sampling affects the spectrum of the reconstructed signal.

This section shows sampling theorem in a real-world context, including how filtering before sampling can reduce the required sampling rate, how under sampling leads to aliasing and distortion, how the Nyquist rate may not provide optimal reconstruction, how oversampling improves reconstruction quality but with diminishing returns, and how there exists an optimal sampling rate that balances accuracy and efficiency.

The analysis reveals how system designers must consider several factors from filtering to reconstruction, when determining appropriate sampling rates.

  - Error and Robustness Analysis:

White Gaussian noise is added to the filtered ECG signal at various levels (30 dB, 20 dB, 10 dB, and 0 dB). The noisy signals are sampled at the previously determined optimal sampling rate, and 
reconstruction is performed on the noisy samples. We then calculate reconstruction errors and compare them across noise levels.

Both time-domain and frequency-domain analyses of the noisy reconstructions are performed and error metrics are visualized to show the relationship between noise level and reconstruction quality.

This section addresses the reality that all practical systems operate in the presence of noise. It demonstrates how noise propagates through the signal processing chain, the impact of noise on sampling and reconstruction, how filtering can mitigate but not eliminate noise effects, and the robustness of different sampling rates to noise

  - Design Insights and Discussion:

This project ties together filtering, sampling, and noise analysis_ key principles of practical signal processing. We saw how careful filter design, balancing noise removal with feature preservation, is essential for sensitive signals like ECG, where a 40 Hz cutoff  removed high-frequency noise without distorting critical details. Sampling tests confirmed the Nyquist rate as a theoretical minimum, but also showed that oversampling improves robustness, especially in noisy environments. The interconnectedness of noise levels, sampling rates, and reconstruction quality revealed the need to design systems with safety margins to handle real-world imperfections. These findings have broad relevance in cardiology, in which experts need to study signals accurately to make proper decisions, which requires having raw data that is not affected by outside noise. In conclusion, signal processing isn’t just about applying formulas, but about making smart design choices that meet real-world needs.

Extra Credit: Tachycardic ECG Analysis:

What it does:  Analyzes how tachycardia (elevated heart rate) affects ECG signals and their optimal processing parameters, connecting physiological changes to signal processing requirements.

Implementation details:

- Processes normal and tachycardic ECG data with significantly different heart rates

- Compares ECG characteristics between normal and tachycardic conditions, including:

  - Heart rate (normal: ~60-100 bpm vs. tachycardia: >100 bpm)

  - Frequency content (shift toward higher frequencies)

  - Signal bandwidth (broadened distribution)

- Applies the optimal sampling rate function to both conditions

- Processes signals through the complete pipeline (filtering, sampling, reconstruction)

- Evaluates the impact of heart rate on required processing parameters

This analysis demonstrates how different conditions alter signal characteristics and processing requirements. It shows:

- The relationship between cardiac physiology and signal processing parameters

- How adaptive signal processing can optimize for varying physiological states

- The importance of condition-specific processing in biomedical applications

Key insights and applications:

- Tachycardia compared to normal heart rate exhibits:

  - Higher frequency content due to faster repetition of cardiac cycles

  - Different spectral energy distribution

- These changes affect optimal processing parameters:

  - Tachycardic ECG signals typically require higher sampling rates

  - Filtering requirements may need adjustment to preserve critical features

  - Reconstruction accuracy at the same sampling rate is often lower for tachycardic signals

- System designers for cardiac monitoring equipment should:

  - Consider adaptive processing based on detected heart rate

  - Potentially implement variable sampling rates for different cardiac conditions

This analysis shows the importance of processing in medical monitoring systems. Biomedical engineering applications must provide accurate results across a range of physiological states.

References:

- ECG simulation using MATLAB

Version 1.0.0.0 (281 KB) by Karthik Raviprakash

This code generates all possible forms of ECG signals with the parameters specified by the user.

- https://www.mathworks.com/help/signal/ref/hamming.html

- GitHub, Claude AI and ChatGPT for figuring out syntax and learning how to code.